Purpose of this Document:

The purpose of this document is to give us a good idea of what needs to be accomplished, so we can approach each task in an organized fashion. This document will not contain any particular hardware design or software design.  Instead it will provide a reference for what needs to be done. Basically everything in this document is ‘what’ not ‘how’.

Project Definition: A functional Lidar.

A spinning LIDAR is a device that scans a field of view and determines the distance and angle of an array of points. To accomplish this task an accurate range finding device is spun at a high speed and sampled frequently. Every time the range finding device is sampled, the device records the exact angle that the point was recorded at. The end result is that after one full rotation there is an array of points generated.  Each point has a distance and an angle. This is considered a single LIDAR scan. A good LIDAR system will have a high scan frequency(number of scans per second), a large number of points per scan, large field of view(possibly 360), and accurate measurements for each point.

FOV

(Point 1)

(Point 2)

(Point n)

Spinning LIDAR Specification:

For the particular LIDAR we are designing we must agree on the level of performance that is required because the hardware and software complexity will be dependant on this.

// these specs are subject to change based on what our group agree on and are capable // of.

| LIDAR SPEC | VALUE |
| --- | --- |
| Scans per Second | 1Hz min |
| Points per Scan | 100 min |
| Distance(Err) | +- 0.25(m) max |
| Angle(Err) | +- 0.1(rad) max |
| FOV | 6.28(rad),360(deg) min |
| Range | 5.0(m) min |

Spinning LIDAR High-Level Design:

The LIDAR will have two main modules.  The Base Module and the Range Finding Module.  The purpose of the Base Module will be to spin the Range Finding Module, read data from the Range Finding Module, and record the angle of each distance reading. Once a single scan has been completed the device will send out the array of data points over a data link. The purpose of the Range Finding Module is to provide range data at a high enough sample rate to keep up with the Base Module.  For design of the Base Module see the BaseModuleDesign document. For design of the Range Finding Module see the Range FindingModuleDesign document.

Range Data

Range Finding Module

Base Module

LIDAR Data

{(Range,Angle), ... }

Constraints:

The constraints of the two modules are dependant on the performance specifications mentioned above, and can be calculated mathematically. Because the specs are subject to change I will not bother calculating the specs for each module.

It is expected that there will be a lot of revisions of the Range Finding Module until we get it to work properly. For this reason it is important that the two modules agree on a data format for sending/requesting range information so that new Range Finding Modules can be swapped in without modifying the Base Module.

Base Module Constraints:

The Base Module will need to be capable of encoding the angle of the Range Finding Module accurately, and spin fast enough to meet the above specifications.

Range Finding Module:

The Range Finding Module must have a fast sample rate, and acceptable accuracy in order to meet the specifications.